SHOE COVER CPE BLUE 4,6 GR

113-8281

CHARACTERISTICS:

Compressed Polyethylene (CPE) film, commonly called Polylatex, is made from a mixture of 65% Low Density Polyethylene (LDPE) and 35% Light Low Density Polyethylene (LLDPE). Polylatex film is very lightweight and is a complex hydrocarbon chain. It is practically impervious; hence it is excellent for containing moisture and dirt. It also repels moisture and oil. Polylatex is non-toxic and odourless. Polylatex has excellent resistance to chemical corrosion and its physical properties and not affected by either chemical or solvents. Since it is a form of plastic material, it possesses excellent strength and it can be easily sealed. It is heavily textured for better grip.

PHYSICAL PROPERTIES

Size						Small

Length (L)					41cm +/- 1cm

Height (H)					15cm +/- 1cm

Hole Opening, Stretched				41cm +/- 1cm

Wall Thickness					2.00mil +/- 0.1mm

Weight						4.6g +/- 8%

Tensile Strength					>30 kg/m2

Elongation 					100 - 250%

Colour 						Blue

PACKING:

100pcs/bag, 10bags/carton case

QUALITY ASSURANCE CRITERIA:

Shoe covers are guaranteed to meet 4.0 AQL level II sampling plan per MIL-std-150E for the following defect criteria:

1.	Broken elastic band				4.	Holes/tears in fabric material

2.	Irregular material					5.	Physical dimensions non-compliance

3.	Dirty						6. 	Not clean

Disclaimer: VWR wants to state that this declaration will not discharge the user from his obligation to ensure the product is suitable for the intended use. -  If there is any doubt ask us.

Date of issue: Feb 2020