| OPTICAL TUBE INFO:. Optical Design; Newtonian Reflector. Aperture; 130mm (5.11"). Focal Length; 650mm (25.59"). Focal Ratio; f/5. Focal Length of Eyepiece 1; 25mm (0.98"). Magnification of Eyepiece 1; 26x. Focal Length of Eyepiece 2; 10mm (0.39"). Magnification of Eyepiece 2; 65x. Barlow Lens; None. Finderscope; StarPointer™ red dot finderscope. Optical Tube; Steel. Highest Useful Magnification; 307x. Lowest Useful Magnification; 19x. Limiting Stellar Magnitude; 13.1. Resolution (Rayleigh); 1.07 arc seconds. Resolution (Dawes); .89 arc seconds. Light Gathering Power (Compared to human eye); 345x. Secondary Mirror Obstruction; 45mm (1.73"). Secondary Mirror Obstruction by Diameter; 34%. Secondary Mirror Obstruction by Area; 11%. Optical Coatings; Glass mirrors coated with aluminum and SiO₂. Optical Tube Length; 635mm (25"). Optical Tube Diameter; 165mm (6.49"). Optical Tube Weight; 8.8 lbs (3.99 kg). Dovetail; CG-5 Dovetail Bar. MOUNT INFO:. Mount Type; Manual Alt-Azimuth. Height adjustment range (includes mount and tripod); Aluminum, 1244.6mm (49") max height. Tripod Leg Diameter; N/A. Accessory Tray; Yes. Tripod Weight; 9.2 lbs (4.17 kg). Slew Speeds; Manual. GPS; Uses phone's GPS. Dovetail Compatibility; CG-5 Dovetail bar. Power Requirements; None (Recommend PowerTank Glow to keep phone charged while using app). Alignment Procedures; Use StarSense Explorer app. Software; StarSense Explorer app ¦ SkyPortal app ¦ Celestron Starry Night Basic Edition Software. Total Kit Weight; 18 lbs (8.16 kg). Included Items; Telescope tube ¦ Mount/tripod (preassembled) ¦ 25mm & 10mm eyepieces ¦ StarPointer finderscope ¦ Accessory tray ¦ StarSense Explorer phone dock. |  |
| --- | --- |